Lab 2

Serial Communications

Objective:

The purpose of this lab is for the student to create a basic network connection between two devices using their serial ports. Students will explore various serial settings using the HyperTerminal program for observation.

Diagram:

Components:

2 – PC’s with serial ports

1 – Null Modem Serial Cable

Procedure:

- To make a basic connection between the PC’s

  - Connect the null modem cable to each PC’s Com Port

  - Start HyperTerminal program. Should be on Desktop. If not, get USB drive from instructor.

  - Enter “513” for the area code. Click OK

  - Click OK for location.

  - Enter a name for the connection. Use “Lab2”

  - Select Connect using Com1

  - Set the bits per second to be 9600

  - Set the data bits to be 8

  - Set the parity to be none

  - Set the stop bits to 1

  - Set flow control to be none

  - Repeat for second PC

- Type a message on one PC and view it on the other

  - Does the message appear on both machines or just one?

_____Just one_____________________

  - Why? Think about our types of communication on slide 6.

____The communication is a half duplex, so it travels one way at a time, but not at the same time ________

- Draw the pinout connections of the null modem serial cable (pin 1 to pin 1). Use Drawing Tools in Word or Copy Paste from Google.

- Make PCA a control PC (settings that do not change) and the other a test PC. Make a connection between the PC’s and observe what happens when settings do not match. (Control PC is set to 9600 baud and test PC is 2400 or control PC is 8 data bits and test PC is 5). Record your observations below. You don’t have to include the exact characters. Explain what you see. (Crazy characters, undesired results, etc.).

Setting That’s Different                                         Effect

Bits per Second                                   ____The characters are completely different; it’s gibberish –

there’s data loss also____

Data Bits                                                _______The characters changed to some other kind of symbols.

On the test PC, the message from the control included new line characters and some data loss.________

Parity                                                     ______The characters, once again, are just gibberish, there is no

indication of what the original data looked like or what it was supposed to be._____

Stop Bits                                                _______The characters are not clear – although it was

interesting to see what the difference one -this time the test pc’s characters were represented by multiple characters on the control as opposed to just being one to one.____

Flow Control                                         _________There was no change in the text; it was clear again___

- Put all settings back to the original settings on both PC’s. Find a file that is around 10MB in size. Send the file from PCA to PCB and record the time it took to send the file over. Send the file again with baud settings of 2400, 4800, and 9600. Record your times below. Don’t wait for the transfer to finish. Wait 10-15 seconds for the time to stabilize, record your time and then cancel. To get your 10 MB file, click start, type “snipping tool”, click the tool to launch. Click New and then use the tool to screen shot your desktop. Click Copy, open MS Word and then paste your screen shot several times (15 or so). Click File, Save As, click file type and change to .doc. You don’t want it to say docx. Check the size of the file. If you need to adjust, either delete or add additional screen shots to the file.

2400                                                    _________10 hours 15 minutes___________________________

4800                                                    _____________5 hours 20 minutes______________

- _______________2 hours 43 minutes______________

- Figure out the time needed to transfer a 2.4Gigabyte file with the following network speeds (remember network speeds are in bits and the file is in bytes). You can use an online file transfer calculator using Google if you prefer.

  - 9.6kb 					___26 days, 16 hr, 47 min, 47 sec __

  - 56kb					_4 days, 13 hr, 51 min, 2 sec_

  - 10Mb					______36 min, 2 sec___

  - 100Mb				_____3 min, 36 sec_____

  - 1Gb					____21 sec________

  - 10Gb					_____2 sec___________

- What did you learn by completing this lab?

_______I learned about the RS232 Protocol with serial connections, more details about connection speed and how the older systems of connection such as serial connections took a very, very long time. Obviously connections with higher network speeds are able to transfer data more readily, but the difference is staggering – weeks get turned into seconds. _______________________________________________________

_________________________________________________________