Lab 12

IPV6

Objective:

The purpose of this lab is for the student to understand, create and implement IPV6. Students will also utilize IOS commands to explore and configure IPV6 on network routers and switches.

Diagram:

Components:

18 – PC’s with an Ethernet NIC

3 – Layer 3 Switches

Procedure:

- Connect all PC’s together and put their static IP addresses, subnet masks and gateways on them.

- Label the Layer 3 switches VLAN IP addresses below

L3 Switch 1

| VLAN | Sw. Port | IP Address |
| --- | --- | --- |
| 1 | 1 | 1111:1111:1111:1111::1/64 |
| 2 | 2 | 1111:1111:1111:2222::1/64 |
| 3 | 3 | 1111:1111:1111:3333::1/64 |
| 4 | 4 | 1111:1111:1111:4444::1/64 |
| 5 | 5 | 1111:1111:1111:5555::1/64 |
| 6 | 6 | 1111:1111:1111:6666::1/64 |
| 10 | 10&11 | 1000::1/64 |

L3 Switch 2

| VLAN | Sw. Port | IP Address |
| --- | --- | --- |
| 1 | 1 | 2222:2222:2222:1111::1/64 |
| 2 | 2 | 2222:2222:2222:2222::1/64 |
| 3 | 3 | 2222:2222:2222:3333::1/64 |
| 4 | 4 | 2222:2222:2222:4444::1/64 |
| 5 | 5 | 2222:2222:2222:5555::1/64 |
| 6 | 6 | 2222:2222:2222:6666::1/64 |
| 10 | 10&11 | 1000::2/64 |

L3 Switch 3

| VLAN | Sw. Port | IP Address |
| --- | --- | --- |
| 1 | 1 | 3333:3333:3333:1111::1/64 |
| 2 | 2 | 3333:3333:3333:2222::1/64 |
| 3 | 3 | 3333:3333:3333:3333::1/64 |
| 4 | 4 | 3333:3333:3333:4444::1/64 |
| 5 | 5 | 3333:3333:3333:5555::1/64 |
| 6 | 6 | 3333:3333:3333:6666::1/64 |
| 10 | 10&11 | 1000::3/64 |

- Label the routing tables for the Layer 3 switch and routers below.

| L3 Switch 1 | L3 Switch 2 | L3 Switch 3 |
| --- | --- | --- |
| C-1111:1111:1111:1111::1/64 | C-2222:2222:2222:1111::1/64 | C-3333:3333:3333:1111::1/64 |
| C-1111:1111:1111:2222::1/64 | C-2222:2222:2222:2222::1/64 | C-3333:3333:3333:2222::1/64 |
| C-1111:1111:1111:3333::1/64 | C-2222:2222:2222:3333::1/64 | C-3333:3333:3333:3333::1/64 |
| C-1111:1111:1111:4444::1/64 | C-2222:2222:2222:4444::1/64 | C-3333:3333:3333:4444::1/64 |
| C-1111:1111:1111:5555::1/64 | C-2222:2222:2222:5555::1/64 | C-3333:3333:3333:5555::1/64 |
| C-1111:1111:1111:6666::1/64 | C-2222:2222:2222:6666::1/64 | C-3333:3333:3333:6666::1/64 |
| C-1000::1/64 | C-1000::2/64 | C-1000::3/64 |
| S-2222:2222:2222::/48 -> 1000::2 | S-1111:1111:1111::/48 -> 1000::1 | S-1111:1111:1111::/48 -> 1000::1 |
| S-3333:3333:3333::/48 -> 1000::3 | S-3333:3333:3333::/48 -> 1000::3 | S-2222:2222:2222::/48 -> 1000::2 |

- Place the switch ports into the proper vlans. Configure all of the switches vlan IP addresses.

- Enable ipv6 routing on the Layer 3 switches

- On switch 1 enable router EIGRP as follows:

  - Make the AS number 1

    - What command did you enter

Ipv6 router eigrp 1

  - Make the EIGRP ID 10.10.10.1

    - What command did you enter

Eigrp router-id 10.10.10.1

  - Turn on EIGRP

    - What command did you enter

No shut

  - Enable EIGRP on the vlans

    - What commands did you enter

int vlan 1

ipv6 eigrp 1

int vlan 2

ipv6 eigrp 1

int vlan 3

ipv6 eigrp 1

int vlan 4

ipv6 eigrp 1

int vlan 5

ipv6 eigrp 1

int vlan 6

ipv6 eigrp 1

int vlan 10

ipv6 eigrp 1

- On switch 2 enable router EIGRP as follows:

  - Make the AS number 1

    - What command did you enter

Ipv6 router eigrp 1

  - Make the EIGRP ID 10.10.10.2

    - What command did you enter

Eigrp router-id 10.10.10.2

  - Turn on EIGRP

    - What command did you enter

No shut

  - Enable EIGRP on the vlans

    - What commands did you enter

int vlan 1

ipv6 eigrp 1

int vlan 2

ipv6 eigrp 1

int vlan 3

ipv6 eigrp 1

int vlan 4

ipv6 eigrp 1

int vlan 5

ipv6 eigrp 1

int vlan 6

ipv6 eigrp 1

int vlan 10

ipv6 eigrp 1

- On switch 3 enable router EIGRP as follows:

  - Make the AS number 1

    - What command did you enter

Ipv6 router eigrp 1

  - Make the EIGRP ID 10.10.10.3

    - What command did you enter

Eigrp router-id 10.10.10.3

  - Turn on EIGRP

    - What command did you enter

no shut

  - Enable EIGRP on the vlans

    - What commands did you enter

int vlan 1

ipv6 eigrp 1

int vlan 2

ipv6 eigrp 1

int vlan 3

ipv6 eigrp 1

int vlan 4

ipv6 eigrp 1

int vlan 5

ipv6 eigrp 1

int vlan 6

ipv6 eigrp 1

int vlan 10

ipv6 eigrp 1

- Verify the routing table on the Layer 3 switches

  - What command did you enter? show ipv6 route

- Can the computers ping each other? no

- Remove EIGRP from all 3 Layer 3 switches and all VLAN interfaces

- On Switch 1 add static routes for 2222:2222:2222::/48 and 3333:3333:3333::/48

  - What commands did you enter

Ipv6 route 2222:2222:2222::/48 1000::2

Ipv6 route 3333:3333:3333::/48 1000::3

- On Switch 2 add static routes for 1111:1111:1111::/48 and 3333:3333:3333::/48

  - What commands did you enter

Ipv6 route 1111:1111:1111::/48 1000::1

Ipv6 route 3333:3333:3333::/48 1000::3

- On Switch 3 add static routes for 1111:1111:1111::/48 and 2222:2222:2222::/48

  - What commands did you enter

Ipv6 route 1111:1111:1111::/48 1000::1

Ipv6 route 2222:2222:2222::/48 1000::2

- Verify the routing table on the Layer 3 switches

  - What command did you enter? show ipv6 route

- Can the computers ping each other? No, but they’re supposed to

- Create the lab diagram below (PCs, router and switch connections) in Packet Tracer or Visio. Label the IP addresses, subnet masks, and gateways used by each device. Label the routing tables next to the switch and router and the switches vlan IP addresses and port vlan information.