Lab 4

EtherChannel and DHCP

Objective:

The purpose of this lab is for the student to understand, create and implement EtherChannel connections and DHCP. Students will also utilize IOS commands to explore and configure EtherChannel connections and DHCP on network routers and switches.

Diagram:

Components:

6 – PC’s with an Ethernet NIC

1 – 3650 Layer 3 Switch

2 – 2960 Layer 2 Switches

2 – 4321 Routers (with NIM 2T WAN card)

Useful commands for this lab:

show ip route – view routing table

show vlan – View vlan table and port assignments

show ip interface brief – view status of layer 3 ports

show access-lists – view access lists and hit counters

show etherchannel summary – view port channel status

Add IP address to Serial Interface (same as Ethernet)

interface Serial0/1/0

ip address 5.5.5.1 255.255.255.0

no shutdown

Enable routing on Layer 3 switch

ip routing

PPP – Clock rate only on DCE side

interface serial0/1/0

encapsulation ppp

clock rate 1536000

ip address 1.1.1.1 255.255.255.0

no shutdown

Assign range of ports to port channel

interface range GigabitEthernet1/0/9-10

channel-group 5 mode on

Configure Port Channel for trunking on L3 Switch

interface Port-Channel 5

switchport trunk encapsulation dot1q

switchport mode trunk

no shutdown

Configure Port Channel for trunking on L2 Switch

interface Port-Channel 5

switchport mode trunk

no shutdown

Configure DHCP on Router

ip dhcp pool netcom

network 1.1.1.0 255.255.255.0

default-router 1.1.1.1

ip dhcp excluded-address 1.1.1.1 1.1.1.2

Configure Port Fast on a port

interface GigabitEthernet 1/0/1

spanning-tree portfast

Configure EIGRP

router eigrp 5

network 1.1.1.0 0.0.0.255

network 11.11.11.0 0.0.0.255

no auto-summary

Procedure:

- Connect all PC’s together and put their static IP addresses, subnet masks and gateways on them. Test connectivity by pinging each machine within the network.

- Use the diagram above to label the Layer 3 switches VLAN IP addresses and ports below

| Vlan | IP Address | Switch Port |
| --- | --- | --- |
| 2 | 2.2.2.1/24 | 2 |
| 3 | 3.3.3.1/24 | 3 |
| 10 | 10.10.10.2/24 | 10 |
| Trunk |  | 21-22 |

- Use the diagram above to label the routing tables for the Layer 3 switch and routers below.

| Router1 | Router2 | Layer3 Switch |
| --- | --- | --- |
| C-1.1.1.0/24 | C-10.10.10.0/24 | C-2.2.2.0/24 |
| C-11.11.11.0/24 | C-11.11.11.0/24 | C-3.3.3.0/24 |
| D-2.2.2.0/24-> 11.11.11.2 | D-1.1.1.0/24-> 11.11.11.1 | C-10.10.10.0/24 |
| D-3.3.3.0/24-> 11.11.11.2 | D-2.2.2.0/24-> 10.10.10.2 | D-11.11.11.0/24-> 10.10.10.1 |
| D-10.10.10.0/24-> 11.11.11.2 | D-3.3.3.0/24-> 10.10.10.2 | D-1.1.1.0/24-> 10.10.10.1 |

- Place the switch ports into the proper vlans. Configure all of the switches vlan IP addresses.

  - Example

interface Gigabitethernet 1/0/1

switchport mode access

switchport access vlan 2

no shutdown

interface vlan 2

ip address 2.2.2.1 255.255.255.0

no shutdown

- Enable routing on the Layer 3 switch

- View the routing table on the Layer 3 switch

- Verify that the PC’s on the switch can ping each other

- Configure the IP addresses on the fastethernet router interfaces and turn the interfaces on

- Plug the DCE cable into the router with the 10.10.10.0/24 and 11.11.11.0/24 networks.

- Plug the DTE cable into the router with the 11.11.11.0/24 and 1.1.1.0/24 networks.

- Make the serial0/0 interfaces on both routers PPP

- Make the serial0/0 interface clock rate 56k on the master

- Place IP addresses on the router serial0/0 interfaces

- Use a dynamic routing protocol (EIGRP or OSPF) on the two routers and Layer 3 switch. Remember: Dynamic routing protocols require a “network” statement for each interface on the device. Also remember the EIGRP autonomous system number should be the same on all devices.

  - Example: Router 1

router eigrp 5

network 1.1.1.0 0.0.0.255

network 11.11.11.0 0.0.0.255

no auto-summary

- View and verify the routing table on both routers and the L3 Switch. Paste below once all 3 L3 devices have all routes.

- Connect 2 interfaces on the Layer 3 switch to 2 interfaces on the Layer 2 switch as the diagram shows

- On the Layer 3 switch create an EtherChannel with the 2 ports that were just connected to the Layer 2 switch. Use the mode of on and a channel-group number of 1. My example above uses group 5. Make sure to adjust the commands to meet these requirements (channel-group 1 mode on) and (interface port-channel 1)

  - What commands did you enter?

Interface range fastethernet0/21-22

Channel-group 1 mode on

  - What is the name of the new interface that was created?

Interface Port-channel1

- Select the new interface and place it into trunk mode

Interface port-channel1

Switchport trunk encapsulation dot1q

Switchport mode trunk

What commands did you enter?

- View the running configuration

  - Are the physical ports in the port channel also in trunk mode? Yes

  - Why? The ports belong to the port channel, so their settings are changed when we change settings for the channel.

- Verify that all 4 PC’s on the Layer 2 and Layer 3 switch can ping each

other

- Run a continuous ping from 2.2.2.10 to 2.2.2.11

  - Example

ping -t 2.2.2.10

The “-t” will send the ping indefinitely or until “Control + C” is pushed or until the Command prompt window is closed.

- Remove one of the links between the Layer 2 and Layer 3 switch

  - Did the continuous ping continue or stop? It continued

  - Why? The connection was not severed; the etherchannel treats both physical ports as one logical port and therefore the “port” did not lose its connection.

- On the router that has the 1.1.1.0/24 network directly connected, create a dhcp pool with the following settings

  - Pool Name: NetCom

  - Network of 1.1.1.0/24

  - Gateway of 1.1.1.1

  - Exclude IP address 1.1.1.1

Ip dhcp pool NetCom

Network 1.1.1.0 255.255.255.0

Default-router 1.1.1.1

Exit

Ip dhcp excluded-address 1.1.1.1

What commands did you enter?

- Disconnect the PC’s 1.1.1.10/24 and 1.1.1.11/24 from the switch. Change them to DHCP clients and plug them back in.  If the PCs are being stubborn, open a command prompt and enter “ipconfig /renew”

  - Did they receive IP addresses? No, it says DHCP failed

  - How long did it take? It looks like a minute or so

- Disconnect the PC’s 1.1.1.10/24 and 1.1.1.11/24 from the switch. Place the switch ports the PC’s plug into to be portfast

Int range fa0/1-2

Sw m a

Spanning-tree portfast

What commands did you enter?

  - Did they receive IP addresses?

Yes

  - How long did it take?

Literally instantly

  - Was this faster than before?

Very much so

  - Why? Because the ports were put into portfast state

- Can every PC ping each other? Yes!

- Create the lab diagram below (PCs, router and switch connections) in Packet Tracer or Visio. Label the IP addresses, subnet masks, and gateways used by each device. Label the routing tables next to the switch and router and the switches vlan IP addresses and port vlan information. Label the port-channel interfaces on the switches.See the diagram at the beginning of the lab for a suggestion on how to do this.